# Discussion Forum Unit 1: Designing a 2-Bit Binary Adder with Logic Gates

Binary addition is one of the clearest ways to see how simple logic gates combine to perform real arithmetic. In this post I design a combinational circuit that adds two 2-bit numbers, A = A1A0 and B = B1B0, using only AND, OR, and XOR gates, and I trace how the inputs flow through the gates to the output.

## Building Blocks: Half Adder and Full Adder

A single-column binary addition follows four rules: 0+0=0, 0+1=1, 1+0=1, and 1+1=10 (a sum of 0 with a carry of 1). A half adder captures this for two bits, using exactly two gates (Ndjountche, 2016): Sum = A XOR B, and Carry = A AND B. The XOR gate outputs 1 only when the inputs differ, matching the sum bit, and the AND gate outputs 1 only when both inputs are 1, which is exactly when a carry is generated.

To add multi-bit numbers, each higher column must also accept a carry coming in from the column below. That circuit is the full adder, which adds three bits (A, B, and Cin): Sum = A XOR B XOR Cin, and Cout = (A AND B) OR (Cin AND (A XOR B)).

## Designing the 2-Bit Adder

To add A1A0 + B1B0, I chain a half adder for the least significant bit with a full adder for the next bit:

Bit 0 (half adder): S0 = A0 XOR B0 and C0 = A0 AND B0.

Bit 1 (full adder): S1 = A1 XOR B1 XOR C0 and Cout = (A1 AND B1) OR (C0 AND (A1 XOR B1)).

The complete result is the three-bit value Cout S1 S0. Although A and B are each 2 bits, their sum can be as large as 3 + 3 = 6, which is 110 in binary and needs three bits. Reporting only S1S0 without the carry-out would misrepresent results such as 2 + 2, so I include Cout as the most significant result bit.

## Truth Table

| A (A1A0) | B (B1B0) | A + B | Cout | S1 | S0 | Result |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (00) | 0 (00) | 0 | 0 | 0 | 0 | 000 = 0 |
| 1 (01) | 1 (01) | 2 | 0 | 1 | 0 | 010 = 2 |
| 2 (10) | 1 (01) | 3 | 0 | 1 | 1 | 011 = 3 |
| 3 (11) | 1 (01) | 4 | 1 | 0 | 0 | 100 = 4 |
| 2 (10) | 2 (10) | 4 | 1 | 0 | 0 | 100 = 4 |
| 2 (10) | 3 (11) | 5 | 1 | 0 | 1 | 101 = 5 |
| 3 (11) | 3 (11) | 6 | 1 | 1 | 0 | 110 = 6 |

## Step-by-Step Signal Flow

Take A = 11 (3) and B = 01 (1). First, A0 = 1 and B0 = 1 enter the half adder: the XOR gate outputs S0 = 0, and the AND gate outputs C0 = 1. Next, A1 = 1, B1 = 0, and the carry C0 = 1 enter the full adder: A1 XOR B1 = 1, and XOR-ing with C0 gives S1 = 0. For the carry-out, (A1 AND B1) = 0 and (C0 AND (A1 XOR B1)) = 1, so the OR gate makes Cout = 1. The output is 100, which is decimal 4, exactly matching 3 + 1.

## Analysis of Circuit Behavior

Categorizing outcomes by binary arithmetic rules, the circuit shows three cases: additions with no carry (small sums such as 1 + 1 = 010), additions that generate an internal carry from bit 0 into bit 1, and additions large enough to overflow two bits and set Cout (such as 2 + 2 and 3 + 3). Because the outputs depend only on the current inputs and not on stored state, this is a purely combinational circuit (Ndjountche, 2016). I built and verified this design in Logisim, confirming each truth-table row by toggling the input pins; the circuit diagram is included below.

## Logisim Circuit

[ Insert Logisim circuit screenshot here ]

Question for the group: Since a 2-bit addition can overflow into a third bit, do you think a well-designed adder should always expose a carry-out line, or are there situations where discarding the overflow is the correct engineering choice?

Word count: 592

# References

Ndjountche, T. (2016). Digital electronics 1: Combinational logic circuits. John Wiley & Sons. https://ebookcentral.proquest.com/