# One Arithmetic Core for Many Number Systems

Nicanor Maswili

Department of Computer Science, University of the People

CS 1105: Digital Electronics & Computer Architecture

Instructor: Muhammad Aligohar Bilal

September 30, 2026

# One Arithmetic Core for Many Number Systems

For a computer that does arithmetic in several number systems, my first decision would be to keep one internal format and treat the others as input and output formats. Decimal, octal, hexadecimal, and BCD are different ways of writing the same values (Ndjountche, 2016a), so the machine needs one good binary core and conversion at the edges.

# An Integrated Approach to the Four Operations

The adder should sit at the center, because the other three operations can be built from it. With numbers stored in two's complement, A minus B equals A plus the inverted bits of B plus 1, so a row of XOR gates on the B input, controlled by a SUB line that also feeds the carry-in, turns one adder into an adder/subtractor (Ndjountche, 2016a, 2016b). Multiplication can be done by shift-and-add, where every 1 bit in the multiplier adds a shifted copy of the multiplicand, and division by shift-and-subtract, restoring the value whenever a trial subtraction goes negative (Ndjountche, 2016b). All four operations then share one adder, a shift register, and a small control unit, and an opcode drives a multiplexer that picks which result goes out. That is basically what an ALU is: one block that performs arithmetic and logic operations and is told which one to run (Learn Computer Science, 2022).

Since every operation passes through the adder, its speed sets the speed of the whole unit. A ripple-carry adder is small, but the carry has to travel through every bit, so for wider words I would use a carry-lookahead adder, which computes generate (AB) and propagate (A XOR B) signals so all carries are ready sooner (Ndjountche, 2016b). The status flags (carry, zero, sign, overflow) also come from the adder, so a compare is really a subtraction whose result is thrown away. For a workload heavy on multiplication I would add a dedicated array multiplier, since shift-and-add costs one clock cycle per bit. The trade-off is chip area against speed.

# The Industry Most Affected: Banking and Finance

I think banking and finance would change the most. The reason is a small detail from the reading on fractions: a value like 0.1 cannot be stored exactly in binary, because it turns into a repeating pattern, the same way 1/3 repeats in decimal (Ndjountche, 2016a). For one calculation the error is tiny, but a bank running millions of interest, tax, and currency conversions a day can end up with totals that are off by a cent, and in finance that is not acceptable. Most software works around this by storing money as whole cents or doing decimal math in software, which is slower.

Now picture processors that handle decimal and binary equally well in hardware, using BCD or the decimal formats that researchers pushed for and the IEEE 754 standard now defines (Cowlishaw, 2003; IEEE, 2019). Money could be calculated exactly and at full hardware speed. Cross-currency payments could be settled and checked in real time. Audits would get simpler because the numbers the machine produces would match what an accountant gets on paper. In regions where mobile money carries a large share of daily payments, exact and fast arithmetic could also push down the cost of very small transactions.

# Why Number Systems and Arithmetic Matter for Advanced Systems

Understanding number systems is what lets a designer make real trade-offs instead of guessing. The representation decides the range, the precision, and how much hardware is needed. Two's complement is a good example: once you understand it, you see why one adder handles both positive and negative values, and why overflow happens when two positive numbers give a negative-looking result (Ndjountche, 2016a). Knowing fixed point versus floating point tells you when a cheap integer unit is enough, which matters in phones and embedded devices with tight power budgets. Hexadecimal matters too, since engineers read memory and machine code in hex, and the Learning Vibes (2023) video presents it mainly as a shorter way to write binary. Modern GPUs and AI chips rely on small 8-bit and 16-bit formats, and that only works because designers know exactly how much precision each format keeps.

Question for the class: If you were building a processor for a bank, would you add a dedicated decimal (BCD) arithmetic unit, or keep a binary-only ALU and store money as whole cents in software? What would you gain and lose with each choice?

Word count: 733

# References

Cowlishaw, M. F. (2003). Decimal floating-point: Algorism for computers. In Proceedings of the 16th IEEE Symposium on Computer Arithmetic (pp. 104-111). IEEE. https://doi.org/10.1109/ARITH.2003.1207666

IEEE. (2019). IEEE standard for floating-point arithmetic (IEEE Std 754-2019). https://doi.org/10.1109/IEEESTD.2019.8766229

Learn Computer Science. (2022, May 31). What is arithmetic logic unit? | ALU in computer architecture explained [Video]. YouTube. https://www.youtube.com/watch?v=H_aoaQYgKT8

Learning Vibes. (2023, January 9). Number system in digital electronics | Introduction to number system | Digital electronics | Mruduraj [Video]. YouTube. https://www.youtube.com/watch?v=XICyh0hO254

Ndjountche, T. (2016a). Digital electronics 1: Combinational logic circuits. ISTE; John Wiley & Sons. https://doi.org/10.1002/9781119318620

Ndjountche, T. (2016b). Digital electronics 2: Sequential and arithmetic logic circuits. ISTE; John Wiley & Sons. https://doi.org/10.1002/9781119329756