# Choosing a Computer Architecture: Matching the Design to the Job

Nicanor Maswili

Department of Computer Science, University of the People

CS 1105: Digital Electronics & Computer Architecture

Instructor: Muhammad Aligohar Bilal

October 14, 2026

# Choosing a Computer Architecture: Matching the Design to the Job

If I were building a presentation comparing computer architectures, my main message would be that there is no "best" architecture, only the best fit for a particular job. Wu (2023) describes computer architecture as the bridge between software and hardware, and the instruction set architecture (ISA) as the contract on that bridge: it defines which instructions, registers, and data types the software can rely on. Choosing an architecture therefore means choosing both the hardware and the software world that comes with it.

# Critical Factors When Selecting an Architecture

1. The workload. The first question is what the system will actually do. A laptop runs many varied programs and needs strong single-thread speed, a web server handles thousands of small requests in parallel and benefits from many cores, and a microwave controller only needs to read buttons and drive a timer. Both (2020) explains that CPU performance depends on how efficiently the fetch, decode, and execute cycle is used, for example by overlapping its stages, and on having several cores that can run threads at the same time, and different workloads benefit from these in different ways.

2. ISA and software compatibility. Software is compiled into machine language for one ISA, and machine code for one processor family will not run on another (Computer Hope, 2019). A company with years of x86 software may find that moving to ARM costs more in rewriting and testing than it saves in hardware.

3. Power efficiency. For phones, laptops, and data centres, performance per watt often matters more than peak speed, because it decides battery life and electricity bills. Simpler RISC designs such as ARM and RISC-V usually do well here.

4. Cost and licensing. Cost includes the chip, the licence, and the long-term running cost. x86 is controlled by two companies, ARM designs are licensed, and RISC-V is an open, royalty-free standard that anyone can implement (RISC-V International, n.d.).

5. Real-time and I/O needs. Embedded systems care about predictable response to hardware events. This week's Z80 readings show how much the interrupt design matters: a simple system can use Mode 1 with one fixed handler, while a system with many devices benefits from vectored Mode 2 (Cook, 2015).

| Architecture | ISA style | Strength | Typical use |
| --- | --- | --- | --- |
| x86 | CISC | Huge software base, high peak speed | Desktops, many servers |
| ARM | RISC | Performance per watt | Phones, Apple Macs, cloud servers |
| RISC-V | RISC, open | No licence fees, customizable | Microcontrollers, research, new chips |
| Z80 | CISC, 8-bit | Simple, cheap, predictable | Classic and embedded controllers |

# The Future of Computer Architecture

I expect three trends to shape the next decade. First, heterogeneous chips: instead of one kind of core, a single chip will combine fast cores, efficient cores, a GPU, and AI accelerators, and software will send each task to the unit that handles it best. Second, chiplets: designers will build processors from smaller dies joined in one package, which improves manufacturing yield and lets companies mix parts. Third, open ISAs such as RISC-V will keep growing, because companies can build custom processors on a shared standard without paying licence fees (RISC-V International, n.d.).

The impact on the industry will be large. Competition will lower costs and weaken the old x86 dominance, especially in servers. Energy efficiency will become a selling point as AI data centres consume more power. For developers, the biggest change is that software must be portable across ISAs, so cross-compiling and testing on more than one architecture will become a normal skill.

# A Real-World Example: AWS Graviton

Amazon Web Services designed its own ARM-based Graviton processors for cloud servers instead of relying only on x86 chips. AWS reports that Graviton instances give the best price performance for a broad range of cloud workloads and use up to 60% less energy than comparable EC2 instances for the same performance (Amazon Web Services, n.d.). For a customer running web servers or databases, moving to Graviton can therefore cut both the monthly bill and the carbon footprint, often with little change to code written in portable languages such as Java or Python. The example shows every factor above working together: a workload that suits many efficient cores, an ISA with good software support, and large savings in power and cost.

Question for the class: Since RISC-V is free to use and customize, do you think it will eventually replace ARM in phones and embedded devices, or will ARM's mature software ecosystem keep it ahead? What would a company need to consider before switching?

Word count: 741

# References

Amazon Web Services. (n.d.). AWS Graviton processors. https://aws.amazon.com/ec2/graviton/

Both, D. (2020, July 7). The central processing unit (CPU): Its components and functionality. Red Hat. https://www.redhat.com/en/blog/cpu-components-functionality

Computer Hope. (2019, June 30). Machine language. https://www.computerhope.com/jargon/m/machlang.htm

Cook, M. (2015, April 15). Z80 interrupts. Z80 Journal. https://z80journal.wordpress.com/2015/04/15/z80-interrupts/

RISC-V International. (n.d.). About RISC-V International. https://riscv.org/about/

Wu, H.-W. (2023, June 22). Computer architecture explained: The bridge between software and hardware. DataSci Ocean. https://datasciocean.com/en/other/what-is-computer-architecture/